# Project P10 Research Notebook

Personalized Deep Learning Model for Closed-Loop 40 Hz Entrainment to Optimize Theta-Gamma Coupling in Alzheimer's Disease

Researcher: Amaar Chughtai

School: Valley Christian High School

Fair: Synopsys Science and Engineering Fair 2026

Date Range: December 10, 2025 - March 3, 2026

# Background Research and Project Selection

December 10, 2025 - Initial Brainstorming

Today I began planning my Synopsys project. I wanted to work with brain signals (EEG) and neurodegenerative disease because I find neuroscience fascinating and there is substantial publicly available data.

Key question: Should I focus on Alzheimer's Disease or Parkinson's Disease?

| Feature | Alzheimer's Disease | Parkinson's Disease |
| --- | --- | --- |
| Primary symptom | Memory loss, cognitive decline | Movement problems (tremor, rigidity) |
| EEG signature | Reduced gamma oscillations (30-80 Hz) | Excessive beta oscillations (13-30 Hz) |
| Prevalence | ~6.7 million Americans (2023) | ~1 million Americans |

Decision: Alzheimer's Disease

Rationale:

- Larger public health impact

- More EEG datasets available

- Recent breakthrough research on gamma enhancement therapy

- Personal connection - my grandmother had dementia

December 11-12, 2025 - Deep Dive into Alzheimer's Biology

Key findings from literature review:

- Gamma oscillations (30-80 Hz) are critical for memory encoding and retrieval

- Generated by PING mechanism (Pyramidal-Interneuron Network Gamma): fast-spiking interneurons synchronize pyramidal neuron firing

- Alzheimer's disrupts gamma: Reduced power and poor synchronization across brain regions

- "Neurons that fire together, wire together" - gamma synchrony enables memory formation

- Phase-Amplitude Coupling (PAC): Theta rhythm (4-8 Hz) modulates gamma amplitude, quantifies coupling strength

December 16, 2025 - The Pivot That Changed Everything

Breakthrough discovery: Found Iaccarino et al. (2016) Nature paper: "Gamma frequency entrainment attenuates amyloid load and modifies microglia."

Key findings from this paper:

- MIT researchers exposed AD mice to 40 Hz flickering light

- Light drove brain to oscillate at gamma frequency (entrainment)

- Triggered microglia to clear amyloid-beta plaques by 40-50%

- Human clinical trials by Cognito Therapeutics showed cognitive improvement

Critical insight: Current protocols use fixed schedules (40 Hz on for 1 hour, same for every patient). But individual responses vary dramatically. Some patients entrain robustly, others show minimal response.

New project idea: Build a closed-loop system that reads EEG in real-time, predicts when the brain will lose gamma entrainment, and delivers stimulation only when needed. Adaptive instead of fixed scheduling.

# Data Acquisition and Technical Development

January 3, 2026 - Dataset Download and BIDS Format Investigation

Downloaded 847 MB dataset from OpenNeuro. Found OpenNeuro ds005048: "40 Hz Auditory Entrainment in Dementia" (Lahijanian et al., 2024).

Dataset specifications:

- 35 elderly subjects from memory clinic in Tehran

- 19 EEG channels (10/20 system), 250 Hz sampling

- Protocol: 40 Hz amplitude-modulated auditory pulses

- Design: 40s stimulation + 20s rest periods

- File format: BIDS-compliant .set/.fdt pairs

January 5-15, 2026 - Data Loading Technical Challenges

Major technical hurdle: EEGLAB .set files use MATLAB v7.3 format with HDF5 backend. Standard Python tools (MNE, EEGLAB readers) failed to load properly.

Solution developed:

- Read .set HDF5 files directly with h5py

- Extract metadata from top-level fields (no EEG wrapper group)

- Read actual data from .fdt files using numpy.fromfile()

- Critical: Reshape with order='F' (Fortran/column-major) due to MATLAB storage format

# Architecture Development and Model Iterations

February 16, 2026 - The Architecture Marathon

Goal: Find optimal architecture for PAC prediction from 2-second EEG windows.

V1 - EEGNet Baseline: Based on Lawhern et al. (2018) - proven architecture for EEG classification. ~1,457 parameters: temporal convolution → depthwise spatial → separable convolution → FC. Result: R² = 0.287 on test set.

V3 - SpecTempNet - CRITICAL DISCOVERY: Combined spectral features with temporal convolution. FEATURE LEAKAGE DETECTED - PAC features directly encoded the target (circular dependency). Inflated R² to 0.999 - completely invalid.

| Model | Parameters | Test R² | Notes |
| --- | --- | --- | --- |
| EEGNet | 1,457 | 0.287 | Baseline architecture |
| SpecTempNet | ~10K | 0.999* | *Feature leakage - invalid |
| Ridge Regression | 427 | 0.315 | Simple linear model |

Key insight: For static PAC prediction from 7 frontal channels, the ceiling appears to be R² ≈ 0.30. This limitation led to the next major development.

# Temporal Prediction and Multiscale TCN Development

February 17, 2026 - Reframing the Problem

Breakthrough realization: Instead of predicting instantaneous PAC, predict future PAC states. This enables proactive control rather than reactive control.

MultiscaleCausalTCN Architecture:

- Input: (batch, T=20, F=73) → Output: future PAC + delta PAC

- Causal depthwise-separable convolutions (no future information)

- Dilation pattern [1,2,4,8] captures multiscale temporal dependencies

- Parameters: ~31,000

# Results and Clinical Validation

February 19, 2026 - Horizon Sweep Analysis

The Critical Experiment: Train separate models to predict PAC at horizons 1-10 seconds. Compare TCN against persistence and Ridge baselines.

Key finding: At 1-2 second horizons, simple baselines suffice. At 5-10 second horizons - the operationally relevant range for proactive control - baselines collapse to negative R² while TCN maintains R² ≈ 0.25.

[FIGURE PLACEMENT: Insert horizon sweep graph here]

February 21, 2026 - Real-Data Validation

| Controller | Alignment | Low-PAC Targeting | High-PAC Rest | PAC Gap |
| --- | --- | --- | --- | --- |
| Fixed Schedule | 45.0% | 61.4% | 28.6% | -6.6 |
| Reactive Threshold | 64.5% | 51.7% | 77.3% | +21.1 |
| TCN Predictive | 72.1% | 82.6% | 61.6% | +30.5 |
| Alignment Oracle | 100.0% | 100.0% | 100.0% | +33.3 |

Statistical significance:

- TCN vs Reactive alignment: 72.1% vs 64.5% (Hedges' g = 1.31, p < 0.001)

- Low-PAC targeting: 82.6% vs 51.7% (g = 4.47, p < 0.001)

- All 35 subjects benefited from TCN controller (binomial p < 0.001)

# Results Summary and Clinical Implications

## Key Findings

- Temporal advantage: TCN maintains R² ≈ 0.25 at 5-10s horizons where baselines fail (negative R²)

- Clinical performance: 72.1% vs 64.5% epoch alignment (p < 0.001, all subjects benefit)

- Targeting precision: 82.6% vs 51.7% stimulation of low-PAC windows (60% improvement)

- Individual differences: 50% of subjects habituate, 50% do not, validating personalized approach

- Efficiency: Reaches 91% of theoretical oracle performance with 10% less stimulation than fixed protocols

## Limitations and Future Work

Current Limitations:

- Single-site dataset: 35 subjects from one clinic in Tehran - generalization needs validation

- Offline analysis: Real-time implementation requires optimization for <1s inference latency

- EEG-only approach: Integration with other biomarkers could improve predictions

## Conclusions

This project successfully developed and validated a predictive closed-loop system for 40 Hz gamma entrainment in Alzheimer's disease. The Temporal Convolutional Network architecture enables 5-second lookahead prediction of brain state, allowing proactive rather than reactive stimulation control.

Clinical impact: The system achieves 72.1% alignment with optimal stimulation timing compared to 64.5% for reactive control, with 82.6% vs 51.7% targeting of therapeutic windows. This represents a 60% improvement in precision while using 10% less total stimulation.

Scientific contribution: First demonstration that future PAC can be predicted from EEG at clinically relevant horizons (5-10 seconds) where traditional baselines fail.

Personalized medicine: Strong individual differences in habituation patterns validate the need for adaptive, personalized control rather than one-size-fits-all protocols.

The results support advancement to clinical trials comparing adaptive vs fixed 40 Hz entrainment protocols for Alzheimer's treatment.

# References

- Iaccarino, H.F., et al. "Gamma frequency entrainment attenuates amyloid load and modifies microglia." Nature 540, 230-235 (2016).

- Tort, A.B., et al. "Measuring phase-amplitude coupling between neuronal oscillations of different frequencies." J. Neurophysiol. 104(2), 1195-1210 (2010).

- Lawhern, V.J., et al. "EEGNet: a compact convolutional neural network for EEG-based brain-computer interfaces." J. Neural Eng. 15(5), 056013 (2018).

- Lahijanian, B., et al. "40Hz Auditory Entrainment" OpenNeuro ds005048 v1.0.1 (2024).

- Canolty, R.T. & Knight, R.T. "The functional role of cross-frequency coupling." Trends Cogn. Sci. 14(11), 506-515 (2010).

Laboratory notebook pages: 120

Analysis period: December 10, 2025 - March 3, 2026